# DVR-TS04D H.264 Network Mini Video Recorder 4CH CCTV DVR HDMI VGA P2P NVR

# 4CH H.264 Network MINI D1 DVR Real-time CCTV Video Recorder P2P DDNS HDMI

Features:

- With H.264 High efficiency Video Compression

- 4CH support 4CH playback at same time,

- 4CH with total 100fps support D1@25fps/ch,

- Support Symbian, S60V3/V5, Windows mobile, iPhone, android, BlackBerry, mobile phone live view via 3G net

-  Support a 1* Laptop HDD, with 2 USB 2.0 slot, support USB, mouse, backup, burning, upgrade

-Strong Network server, support IE review and CMS (can manager over 500 units DVR)

- With Chinese, Russian, English, Italian, Portuguese, Spanish, Arabic 25 language

- Build-in 3322.org, dyndns.net, oray.net, myq-see.com free DDNS.

- Build-in own free DDNS

-Support VGA High Definition output (1280 * 1024), support widescreen LCD (1440*900)

- With P2P Cloud function, WITH HDMI

Specifications:

|  | Model | DVR-TS04D |
| --- | --- | --- |
| System | Main Processor | Hisilicon Hi3515A chip |
|  | Menu display | 1/4 screen; |
| Video | Video Standards | PAL（625 line，50 f/s）；NTSC（525 line，60 f/s） |
|  | Surveillance quality | PAL，D1（704×576)；NTSC，D1（704×480） |
|  | Code ability | 100/120fps 4D1@25fps |
|  | Motion Detection | Zones:396(22*18)detection zones, many levels of Sensitivity setting |
| Record & Playback | Recording Mode | Manual>Alarm>Dynamic Detection>Timing |
|  | Playback | One channel or multi-channel playback (16 channel max synchronized playback) |
|  | Search Mode | Time/Date,Alarm, Motion Detection & exact search(accurate to second) |
| Storage & Backup | Space Occupation | CIF:5G/DAY/CH D1: 20G/DAY/CH |
|  | Video storage | Laptop Hard disk, , and net |
|  | Backup Mode | Network burn, mobile Hard disk and USB burn |
| Interface | Video Input | 4channel, BNC, 1.0Vp-p,75ω |
|  | Video Output | 1channe，1channe |
|  | Audio Input | 1 channe RCA |
|  | Audio Output | 1channe RCA |
|  | Alarm Input | NO |
|  | Alarm output | NO |
|  | Net Interface | RJ-45 (10M/100M) |
|  | PTZ Control | VGA |
|  | USB Interface | 2*USB |
|  | Hard Disk | 1 SATA ports |
| Other | Power Supply | DC5V |
|  | Dimension | 21.7*11.4*4.4 cm (W*D*H) |
|  | Net Weight | 346g |

Package Contents:

1* Mini Network Video Recorder

1* Power Adapter

1* Mouse

1* CD